# Akrometrix Test Services Sample Information Form

Shadow Moiré is the most common choice for warpage measurement, being adaptable to a wide variety of sample types. It is a non-contact, full field optical technique with resolution down to 0.5 micron. Measurements on continuous samples have a maximum field of view of 600 x 600 mm and minimum of 2 x 2 mm. It is the only choice for regions of interest above 240 x 192 mm. Testing may require high temperature white paint based on sample surface optical quality and would be considered destructive in that case. Testing performed with no paint applied would be nondestructive. See below for examples of 3D surface plots and Warpage vs. Temp. plot.

Example applications: BGA, LGA, Flip chip, PoP, QFP, QFN, CSP, TSOP, MLF, etc.

Digital Fringe Projection is a non-contact, full field optical technique with resolution down to 2.5 microns, capable of measuring non continuous surfaces, such as step heights or data islands. DFP measurements are high density, and a variable field of view is provided from 240 x 192 mm down to 45 x 36 mm. Testing may require high temperature white paint based on sample surface optical quality and would be considered destructive in that case. Testing performed with no paint applied would be nondestructive.

Example applications: Connectors, Sockets, Assembled modules, PCBA local areas

Digital Image Correlation is a non-contact, full field optical technique capable of performing in-plane strain/CTE measurements. Out-of-plane deformation is accounted for during in-plane analysis. Measurements are limited to a maximum 75 x 75 mm field of view, but sample size can be up to 400 x 400 mm. Samples must be coated with black and white speckle pattern. As a result, DIC testing should be considered destructive to some extent. See below for an example average X and Y CTE vs. Temp. plot and X and Y strain plots.

# Customer Information

| Company Name: |  |  |  |
| --- | --- | --- | --- |
| Shipping Address: |  |  |  |
| Contact Name: |  |  |  |
| Title: |  | Department: |  |
| Phone Number: |  | Email Address: |  |

| Data Delivery Contact Name: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Title: |  |  | Department: |  |
| Phone Number: |  |  | Email Address: |  |

| Purchasing Contact Name: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Title: |  |  | Department: |  |
| Phone Number: |  |  | Email Address: |  |

| Project Name: |  |
| --- | --- |
| Results Requested by Date: |  |

Samples Return: Because additional testing is sometimes desired, Akrometrix will wait to return samples until receiving customer confirmation, unless "Immediate Return" is selected.

| Return | Immediate Return | Destroy |
| --- | --- | --- |

| Shipper: | UPS, FedEx, etc. |
| --- | --- |
| Account Number: |  |
| Preferred Delivery Speed: | Ground, 3 day air, overnight, etc. |

- Which displacement measurement technique is being requested? (Check all that apply)

| Shadow Moiré | Digital Fringe Projection | Digital Image Correlation |
| --- | --- | --- |

For each requested technique, please fill out the corresponding section below with sample information, test plan details, and results specifications.

*Standard deliverables are highlighted in blue.

*Additional charges apply to sections highlighted in red.

# Interface Analysis

Interface Analysis plots can be applied to the measurement data of two mating surfaces taken with TherMoiré or Fringe Projection systems. Mating surfaces are automatically oriented and plotted in the same window to perform gap analysis under real world conditions. When multiple samples are measured from each attach surface, statistical analysis can be done on the quantity of measured surfaces. Results are organized by like temperatures.

- If measuring mating surfaces, would quantifying the resulting gap between them be useful?

Akrometrix Interface Analysis software can analyze two mating surfaces to perform statistical gap analysis.

| Yes | No |
| --- | --- |

- Will multiple samples of bottom and top surfaces be measured?

If yes, choose Data types to be presented below.

| Yes | No |
| --- | --- |

- What plot types should be presented for surfaces?

Standard deliverables is the 3D Plot. 3D data can be requested in the subsequent section. Additional deliverables can be provided at additional cost.

|  | Plots |
| --- | --- |
| 3D |  |
| Diagonals |  |
| Horizontal Centerline |  |
| Horizontal Edges |  |
| Vertical Centerline |  |
| Vertical Edges |  |
| Gauge vs Temperature |  |

- Any other details?

# Shadow Moiré Sample Information

| Sample(s) Description: | Please be as detailed as possible and attach images for clarification where necessary |
| --- | --- |
| Sample Dimensions: |  |
| Sample Quantities: |  |
| Surface(s) to be measured: | Example: top, bottom, mold compound, solder ball, etc. |
| Name and Location of Local Areas of Interest: | Attach pictures if possible |

- Do we need to adhere to ESD sensitivity standards while testing?

If yes, samples must be matte and light enough in color to obtain good data without optical coating for this to be possible. Additional charges apply.

| Yes | No |
| --- | --- |

- May we apply an optical coating to maximize diffuse reflection of visible light from sample?

If no, samples must be matte and light enough in color to obtain good data without optical coating for measurement to be possible.

| Yes | No |
| --- | --- |

- Does the desired measurement surface of any of the samples have solder balls?

Additional charges apply.

| Yes | No |
| --- | --- |

- If yes, may we remove the solder balls?

Samples cannot be measured on the solder ball surface without first removing the solder balls. Additional charges apply for solder ball removal at Akrometrix.

| Yes | No |
| --- | --- |

- Do any of the samples require a pre-bake?

If yes, turnaround time will increase.

| Yes | No |
| --- | --- |

  - If yes, please specify the pre-bake temperature and time:

Example: 125℃ for 12 hours

  - If no, will parts be shipped in moisture controlled packaging, not to be opened until just before testing?

| Yes | No |
| --- | --- |

- What is the maximum time between opening the moisture controlled packaging and testing the sample(s)?

Example: 30 minutes at 80 % RH

- If any samples to be tested are PCB’s, how should they be supported in the test chamber?

Edge support or area support are typical. Please specify what edges of the sample should be supported. Pictures are helpful.

- Please specify the temperature profile, if needed:

Example: time/temperature pairs, temperature points and rates, or attach a production reflow oven output (either data or graph). Akrometrix can match most production reflow processes up to 2 ℃ /sec, depending on sample thermal mass.

- Please specify the temperatures at which measurements will be taken.

Standard pricing includes 10 temperature points, but more can be added at additional cost.

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |

# TherMoiré Results Specifications

- Z (Out-of-plane) Displacement Units?

| Microns | Mils |
| --- | --- |

- What deliverables are being requested?

Standard deliverables are 3D Graphs and a Warpage vs. Temp. Plot. If Data is selected, it will be provided for any Plot type that is requested.

|  | Plots | Data |
| --- | --- | --- |
| 3D |  |  |
| Diagonals |  |  |
| Horizontal Centerline |  |  |
| Horizontal Edges |  |  |
| Vertical Centerline |  |  |
| Vertical Edges |  |  |
| Warpage vs. Temp. Plot |  |  |

- What scale should be used?

Examples: same for all parts, same for each lot, -100 to 100 µm, etc.

- How should the report be organized?

Default is showing each graph type in order of the measurement’s location along the temperature profile per sample part.

Examples: 3D and diagonals side by side on a page, each part by temperature, etc.

- Any other details?

# Digital Fringe Projection Sample Information

| Sample(s) Description: | Please be as detailed as possible and attach images for clarification where necessary |
| --- | --- |
| Sample Dimensions: |  |
| Sample Quantities: |  |
| Surface(s) to be measured: | Example: top, bottom, mold compound, solder ball, etc. |
| Name and Location of Local Areas of Interest: | Attach pictures if possible |

- Do we need to adhere to ESD sensitivity standards while testing?

If yes, samples must be matte and light enough in color to obtain good data without optical coating for this to be possible. Additional charges apply.

| Yes | No |
| --- | --- |

- May we apply an optical coating to maximize diffuse reflection of visible light from sample?

If no, samples must be matte and light enough in color to obtain good data without optical coating for measurement to be possible.

| Yes | No |
| --- | --- |

- Does the desired measurement surface of any of the samples have solder balls?

Additional charges apply.

| Yes | No |
| --- | --- |

- If yes, may we remove the solder balls?

Samples cannot be measured on the solder ball surface without first removing the solder balls. Additional charges apply for solder ball removal at Akrometrix.

| Yes | No |
| --- | --- |

- Do any of the samples require a pre-bake?

If yes, turnaround time will increase.

| Yes | No |
| --- | --- |

  - If yes, please specify the pre-bake temperature and time:

Example: 125℃ for 12 hours

  - If no, will parts be shipped in moisture controlled packaging, not to be opened until just before testing?

| Yes | No |
| --- | --- |

- What is the maximum time between opening the moisture controlled packaging and testing the sample(s)?

Example: 30 minutes at 80 % RH

- If any samples to be tested are PCB’s, how should they be supported in the test chamber?

Edge support or area support are typical. Please specify what edges of the sample should be supported. Pictures are helpful.

- Please specify the temperature profile, if needed:

Example: time/temperature pairs, temperature points and rates, or attach a production reflow oven output (either data or graph). Akrometrix can match most production reflow processes up to 2 ℃ /sec, depending on sample thermal mass.

- Please specify the temperatures at which measurements will be taken.

Standard pricing includes 10 temperature points, but more can be added at additional cost.

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |

# Digital Fringe Projection Results Specifications

- Z (Out-of-plane) Displacement Units?

| Microns | Mils |
| --- | --- |

- What deliverables are being requested?

Standard deliverables are 3D Plots and a Full Field Signed Warpage Gauge Report. If Data is selected, it will be provided for any Plot type that is requested.

|  | Plots | Data |
| --- | --- | --- |
| 3D |  |  |
| Diagonals |  |  |
| Horizontal Centerline |  |  |
| Horizontal Edges |  |  |
| Vertical Centerline |  |  |
| Vertical Edges |  |  |
| Warpage vs. Temp. Plot |  |  |

- What scale should be used?

Examples: same for all parts, same for each lot, -100 to 100 µm, etc.

- How should the report be organized?

Default is showing each graph type in order of the measurement’s location along the temperature profile per sample part.

Examples: 3D and diagonals side by side on a page, each part by temperature, etc.

- Any other details?

# Digital Image Correlation (DIC) Sample Information

| Sample(s) Description: | Please be as detailed as possible and attach images for clarification where necessary |
| --- | --- |
| Sample Dimensions: |  |
| Sample Quantities: |  |
| Surface(s) to be measured: | Example: top, bottom, mold compound, solder ball, etc. |
| Name and Location of Local Areas of Interest: | Attach pictures if possible |

- Does the desired measurement surface of any of the samples have solder balls?

Additional charges apply.

| Yes | No |
| --- | --- |

- If yes, may we remove the solder balls?

Samples cannot be measured on the solder ball surface without first removing the solder balls. Additional charges apply for solder ball removal at Akrometrix.

| Yes | No |
| --- | --- |

- Do any of the samples require a pre-bake?

If yes, turnaround time will increase.

| Yes | No |
| --- | --- |

  - If yes, please specify the pre-bake temperature and time:

Example: 125℃ for 12 hours

  - If no, will parts be shipped in moisture controlled packaging, not to be opened until just before testing?

| Yes | No |
| --- | --- |

- What is the maximum time between opening the moisture controlled packaging and testing the sample(s)?

Example: 30 minutes at 80 % RH

- If any samples to be tested are PCB’s, how should they be supported in the test chamber?

Edge support or area support are typical. Please specify what edges of the sample should be supported. Pictures are helpful.

- Please specify the temperature profile, if needed:

Example: time/temperature pairs, temperature points and rates, or attach a production reflow oven output (either data or graph). Akrometrix can match most production reflow processes up to 2 ℃ /sec, depending on sample thermal mass.

- Please specify the temperatures at which plots will be provided.

Standard pricing includes 6 temperature points, but more can be added at additional cost.

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |

# DIC Results Specifications

- Strain Units?

Microstrain is most common and used for comparison and calculation of CTE.

| Microstrain | Unity | Percent |
| --- | --- | --- |

- What deliverables are being requested?

Standard deliverable is an average X and Y CTE plot. Additional deliverables can be provided at additional cost. If Data is selected, it will be provided for all Plot types available.

|  | Plots | Data |
| --- | --- | --- |
| X Displacement |  |  |
| Y Displacement |  |  |
| X Strain |  |  |
| Y Strain |  |  |
| Shear Strain |  |  |
| Principal Strain 1 |  |  |
| Principal Strain 2 |  |  |
| Average X and Y CTE Calculation |  |  |

- What scale should be used?

Examples: same for all parts, same for each lot, -100 to 100 µm, etc.

- How should the report be organized?

Default is showing each graph type in order of the measurement’s location along the temperature profile per sample part.

Examples: 3D and diagonals side by side on a page, each part by temperature, etc.

- Any other details?